Zt_Visa_Epp_DLL  interface specification

| Division | Software division |
| --- | --- |
| File purpose | Zt_Visa_Epp DLL development file |
| Subject |  |
| File NO. | ZT_VISA_EPP_V 2.0 |
| File version NO. | V2.0 |
| Date | 2004/09/23 |
| Final revision date | 2005-9-29 |
| Prepared by | 朱平辉 |
| Project owner |  |
| Approved by |  |
| Project Name |  |

contents

Zt_Visa_Epp_DLL  interface specification	1

contents	2

1.	DLL usage	3

2.	DLL interface specifications	3

2.1	Zt_Visa_Epp_dll interface methods	3

2.1.1	Function ——Initialise the serial	3

2.1.2	Function ——Close the serial	4

2.1.3	Function ——Set user information	4

2.1.4	Function ——Read user information	5

2.1.5	Function ——Set control code	5

2.1.6	Function ——Initialisation certification	6

2.1.7	Function —— Get random data	6

2.1.8	Function ——Bi-directional certification	7

2.1.9	Function ——Set the key value	7

2.1.10	Function —— Cleartext key download	8

2.1.11	function —— Ciphertext key download	8

2.1.12	function ——Start the PIN input	9

2.1.13	function ——Get PIN Ciphertext	9

2.1.14	function ——Receive one character from the serial port	10

2.1.15	function——MAC algorithm	10

2.1.16	function ——Data encryption /decryption	10

2.1.17	function ——Software  encryption /decryption	11

2.1.18	function ——IC Card Control Command	12

2.1.19	function ——IC Card Access Command	12

3.	Function return value table	12

# DLL usage

Zt_Visa_Epp used for the entry into the clear text and encryption text The dynamic library for Zt_Visa_Epp called “VISA_EPP_ZT_V2.dll”.

Here ‘s how to use the DLL In Zt_Visa_Epp operation：

1. Use the PinPad_EnablePort to open the keypad serial no.

2. Use the related functions to implement the related functions;

3. Finally use the function of PinPad_DisablePort to close the serial, that is ok.

# DLL interface specifications

## Zt_Visa_Epp_dll interface methods

### Function ——Initialise the serial

| Function description |  | Open the communication port |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Function name |  | int WINAPI PinPad_EnablePort(char *pszCom, long lBoundRate,char *pParity, char *pBite, char *pStopBite) |  |  |  |  |  |
| Property | Variable name |  | type | Variable definition | remarks |  |  |
| Entrance interface parameter | pszCom |  | char * | Serial No | Com1: 1, Com2: 2, …, Com10:10, …. |  |  |
| Entrance interface parameter | lBoundRate |  | long | Baud rate | Support 9600. |  |  |
| Entrance interface parameter | pParity |  | char * | Odd/ even | “N”: no verification, “O”: odd verification, “E”: even verification support no verification |  |  |
| Entrance interface parameter | pBite |  | char * | Data bit | “7”: 7 data bit, “8”: 8 data bit, support 8 data bit . |  |  |
| Entrance interface parameter | pStopBite |  | char * | Stop location | “2”:2 stop bit, “1”:1 stop bit, support 1 stop bit. |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Returned value |  |  | int | See the functions return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function ——Close the serial

| Function description |  | Close the communication port |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_DisablePort() |  |  |  |  |  |
| Property | Variable name |  | type | Variable definition | remarks |  |  |
| Entrance parameter | None |  |  |  |  |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Return value |  |  | int | See the function return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function ——Set user information

| Function description |  | set user information |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_SetCustInfo(char *pszCustInfo, int iLen) |  |  |  |  |  |
| property | Variable name |  | Type | Variable definition | Remarks |  |  |
| Entrance parameter | pszCustInfo |  | char * | 用户信息 | 1---50 bytes data for User information. |  |  |
| Entrance parameter | iLen |  | int | User information length | 1---50 (decimal) |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Return value |  |  | int | See the function return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function ——Read user information

| Function description |  | Read user information |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_GetCustInfo(char *pszData) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | None |  |  |  |  |  |  |
| Exit parameter | pszCustInfo |  | char * | User information |  |  |  |
| Return value |  |  | int | See the function return value table |  |  |  |
| remarks： the first fifty character is user information ，and the 48 character behind them is keyboard key value ,including 16 keystoke value on the master pinpad and the 8 function extendability keystoke value，the 20 character behind them is control code area，the 8 character behind is function description code，the 6 character behind is item type，the 3 character behind is hardware version no., the 3 character behind is firmware version no.，the 4 character behind is production date,， the 4 characters behind is sequence no. in production 。 |  |  |  |  |  |  |  |

### Function ——Set control code

| Function description |  | Set control code |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_SetControlCode(char *pszCodeValue, int nLen) |  |  |  |  |  |
| Property | Variable name |  | Variable type | Variable definition | Remarks |  |  |
| Entrance parameter | pszCodeValue |  | char * | Control code | Control code at 1 character，control value at 2 characters |  |  |
| Entrance parameter | iLen |  | int | Control code length |  |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Return value |  |  | int | See the function return value table |  |  |  |
| remarks： Control Code and control value (1+2 byte) Description 0=CONTRL 30/31/32/33/34/35/36/37/38/39/3A/3B/3C/3D/3E/3F 1= INVKEY 00~7F/80~FF 2= PINOVTM 00~FF 3= PINFILL 00~FF 4= PINMIN 30～39 5= PINMAX 30～39 6= COMCFG 30/37，41/42/43/44/45/46/47/48 7= SPEEK 00～FF 8= BEEPLN 00/01～FF 9= MSGLOK 30～38 A= PRSOVTM 10～1F B= ClEAR FUN 20/21/22 |  |  |  |  |  |  |  |

### Function ——Initialisation certification

| Function description |  | initialisation certification |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_InitAuthorization (char *pszAuthorizedValue, int iLen, char *pszRevData) |  |  |  |  |  |
| Property | Variable name |  | Variable type | Variable definition | remarks |  |  |
| Entrance parameter | pszAuthorizedValue |  | char * | User information data | 16 bytes user information data (UID) |  |  |
| Entrance parameter | iLen |  | int | User information data length |  |  |  |
| Exit parameter | pszRevData |  | char * | Returned certification information |  |  |  |
| Return value |  |  | int | See the function return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function —— Get random data

| Function description |  | Get random data |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_GetChallenge(char *pszRevData) |  |  |  |  |  |
| Property | Variable name |  | Variable type | Variable definition | remarks |  |  |
| Entrance parameter | None |  |  |  |  |  |  |
| Exit parameter | pszRevData |  | char * | Returned random data | 16 bytes random data |  |  |
| Return value |  |  | int | See the function return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function ——Bi-directional certification

| Function description |  | bi-directional authorization |  |  | Prepared by |  | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_BidirectionalAuthorization(char *pszAuthorizedValue, int iLen, char *pszKeyIndex, char *pszRandomData, char *pszRevData) |  |  |  |  |  |
| property | Variable name |  | Type | Variable definition |  | remarks |  |
| Entrance parameter | pszAuthorizedValue |  | char * | Data for initialisation Authorization |  | 16 bytes UID |  |
| Entrance parameter | iLen |  | int | Random data length |  |  |  |
| Entrance parameter | pszKeyIndex |  | char * | Key index |  | Range: 00h~7Fh |  |
| Entrance parameter | pszRandomData |  | char * | random data for Certified |  | 16byte random data by Get random data. |  |
| Exit parameter | pszRevData |  | char * | Returned authorization information |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function ——Set the key value

| Function description |  | Set the key value |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | i int WINAPI PinPad_SetKeyboardValue(char *pszKeyboardValue, int iLen) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszCodeValue |  | char * | Key value data | 16 main key value and 8 extended key values(hex-decimal). |  |  |
| Entrance parameter | iLen |  | int | Key value data length |  |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function —— Cleartext key download

| Function description |  | Cleartext key download |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_DownloadKey(char *pszKeyValue, int iLen, char *pszRevData) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszKeyValue |  | char * | cleartext key | Save the key index, combine the code, key length, clear text key (vary according to key combination methods) |  |  |
| Entrance parameter | iLen |  | int | Key length | 1=8 bytes for 1 Key, 2=16 bytes for 2 Key, 3=24 bytes for 3 Key. |  |  |
| Exit parameter | PszRevData |  | char * | Key verification value | Current Combined key IMK encryption 8 byte “0”: the left 8 of the result. |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： key index——00~7F combine the code——30~34/40~43 key length——1/2/3 clear text key——16/24/32/48/64/96/144 byte data. 0 means to input IMK part manually from EPP. |  |  |  |  |  |  |  |

### Function —— Ciphertext key download

| Function description |  | Ciphertext key download |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_DownloadEncryptKey(char *pszKeyValue, int iLen, char *pszRevData) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszKeyValue |  | char * | Ciphertext Key | Decryption Key index, save the Key index, Key tag, key length, Ciphertext key, Key verification value. |  |  |
| Entrance parameter | iLen |  | int | Key length | 1=8 bytes for 1 Key, 2=16 bytes for 2 Key, 3=24 bytes for 3 Key. |  |  |
| Exit parameter | pszRevData |  | char * | Key verification value | Current TMK/TPK/TAK/TDK/TDEK/TDDK key key encryption 8 byte “0”: the left 8 of the result. |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： Decryption key index——00~7F save the Key index——00~7F Key tag——30~39 key length——1/2/3 Ciphertext key——16/24/32 byte data. Key verification value——8 byte Ciphertext data. |  |  |  |  |  |  |  |

### Function——Start the PIN input

| Function description |  | start the pin input |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_EnablePin(char *pszPinValue, int iLen) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszPinValue |  | char * | Start the PIN parameter | Key index, PIN format code, Audio prompts code, PIN length (when the format is ISO, the CARD no remains). |  |  |
| Entrance parameter | iLen |  | int | PIN parameter length |  |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： Key index——30~7F PIN format code——20/21/23/30/40 Audio prompts code——0/1/2 PIN length——0/1~12 CARD no——12 byte numeric.if PIN format code=20/23. |  |  |  |  |  |  |  |

### Function ——Get PIN Ciphertext

| Function description |  | Get PIN Ciphertext |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_GetPinValue (char *pszCmd, int iLen,int iTimeOut,char *pszPinValue) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance Parameter | pszCmd |  | char* | GetPin Command |  |  |  |
| Entrance Parameter | iLen |  | int | GetPin Command Lenth |  |  |  |
| Entrance Parameter | iTimeOut |  | int | Wait Input TimeOut |  |  |  |
| Exit parameter | pszPinValue |  | char * | Password Ciphertext |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function ——Receive one character from the serial port

| Function description |  | Receive one character form the serial port |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_ReadOneByte( char *pchBuf ) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance Parameter | None |  |  |  |  |  |  |
| Exit parameter | pchBuf |  | char * | Received characters |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function——MAC algorithm

| Function description |  | MAC algorithm |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_MakeMac(char *pszMakeMacData, int iLen, char *pszRevData) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszMakeMacData |  | char * | Need to calculate MAC data | Key index, MAC code type, MAC data |  |  |
| Entrance parameter | iLen |  | int | Need to calculate MAC data length |  |  |  |
| Exit parameter | pszRevData |  | char * | MAC data |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： Key index——30~7F MAC code type——1~3 MAC data——5~1024 byte data. |  |  |  |  |  |  |  |

### Function ——Data encryption /decryption

| Function description |  | data encryption /decryption |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_EncryptData(char *pszEncryptData, int iLen, char *pszRevData) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszEncryptData |  | char * | Data which need to encrypted or decrypted | Key index, Des mode code,encrypted data |  |  |
| Entrance parameter | iLen |  | int | Data length which need to encrypted or decrypted |  |  |  |
| Exit parameter | pszRevData |  | char * | Data which have been encrypted or decrypted |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： Key index——30~7F Des mode code——31/32/41/42 encrypted data——5~1024 byte data. |  |  |  |  |  |  |  |

### Function ——Software  encryption /decryption

| Function description |  | software encryption /decryption |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_TriUntriDes( char *DataInput,char *key, char *DataOutput,int nMode ) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | DataInput |  | char * | Data which need to encrypted or decrypted | Must be HEX，8、16、24-bytes |  |  |
| Entrance parameter | key |  | char * | Key | 8、16、24 bytes |  |  |
| Exit parameter | DataOutput |  | char * | Data which have been encrypted or decrypted | 8、16、24 bytes |  |  |
| Exit parameter | nMode |  | int | Mode | 0：encryption ，1：decryption |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： |  |  |  |  |  |  |  |

### Function—— Display

| Function description |  | Display data on LCD |  |  |  | Prepared by | 朱平辉 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name |  | int WINAPI PinPad_DispInfoLcd(char *pszInfo, int iLen) |  |  |  |  |  |
| Property | Variable name |  | Type | Variable definition | remarks |  |  |
| Entrance parameter | pszInfo |  | char * | display data | Disaplay mode and display data. |  |  |
| Entrance parameter | iLen |  | int | Length of display data |  |  |  |
| Exit parameter | None |  |  |  |  |  |  |
| Return value |  |  | int | See the return value table |  |  |  |
| remarks： Disaplay mode=30 Original code display(1~32 byte display data) Disaplay mode=31 Code display at line 1 (code as 0~9,A/B) Disaplay mode=32 Code display at line 2 (code as 0~9,A/B) Disaplay mode=40~7F Encryption display (8/16 display Encryption data) |  |  |  |  |  |  |  |

# Function return value table

| Reject code | Reject reason |  | Reject code | Reject reason |
| --- | --- | --- | --- | --- |
| 0 | Ok |  | 0xE0 | Low battery |
|  |  |  | 0xE1 | Not original key which can’t download the master key |
| -1 | Communication port can’t be opened |  | 0xE2 | Not master key which can’t download the working key |
| -2 | Error in closing the serial |  | 0xE3 | Wrong key length |
| -3 | Wrong parameter fed into |  | 0xE4 | Key index doesn’t exist |
| -4 | Excessive time in reading data |  | 0xE5 | Unmatched tag or Key index doesn’t exist |
|  |  |  | 0xE6 | Wrong key odd or even verification |
|  |  |  | 0xE7 | Not identical verification value |
|  |  |  | 0xE8 | Wrong command length |
|  |  |  | 0xE9 | Illegal data |
|  |  |  | 0xEA | Coding information doesn’t exist |
|  |  |  | 0xEB | Wrong parameter type |
|  |  |  | 0xEC | Authorization failure |
|  |  |  | 0XED | Authorization get locked |
|  |  |  | 0xEE | Excessive time in the keyboard input |
|  |  |  | 0xEF | Wrong discrepancy |